# IT8212 External BIOS History

| ## Version | ## History | ## Date |
| --- | --- | --- |
| 1.0 | - First release | 09/25/’02 |
| 1.1 | - Change UI. - Add bootable array option. - User changes RAID mode while array rebuilding, force reboot when leaving BIOS. - Add PnP header and PCI header - Reset controller after getting base address. - Add <Ctrl-E> as another hot key to enter Setup Utility. | 11/14/’02 |
| 1.2 | - Reboot system if user changes rebuilding array to other RAID type. - Show each drive's model name and Array/IDE No on screen. - R0 and R0+1 stripe size default 64k - Add BCV entry | 12/09/’02 |
| 1.3.1.3 | - bug fix for 動態磁碟 | 02/18/’03 |
| 1.3.1.31 | - Bug fixed: hard disk boot failure (INT13h ah=0 fail) | 02/24/’03 |
| 1.3.1.4 | - When IBM IC35L0xxxxxx HDD is slave and single, avoid reset controller - If error bit still on after drive select, send command anyway. - Add 4 seconds delay before first time access HDDs. | 03/26/’03 |
| 1.4.1.5 | - Return ax=110h instead of 100h when leaving BIOS initial routine. - Bug fixed: DMA transfer failure on certain main boards. | 06/09/’03 |
| 1.4.1.6 | - Setup Utility can’t be displayed when working with Phoenix BIOS - Always boot from 8212 HDD when using Phoenix BIOS | 07/22/’03 |
| 1.5.1.7 | - Force reconstruction restarts if data inconsistent is found. - Include the replacement drive into array if user press <Y> key. | 10/01/’03 |
| 1.6.1.8 | - If BIOS fails to get rebuild status (error code 51h), show message "Rebuilding is stopped by target drive write error or source drive read error. Please turn off the power and replace disk. Press ANY KEY to continue." instead of "fail to get rebuild status" - PCI configuration space offset 51h bit 0: 0.. Show copyright and other info, 1.. Do not show copyright and drive info - Bug fixed: for Gigabyte VIA chipset MB, fail to boot from secondary HDD. | 02/25/’04 |
| 1.7.1.92 | - Arrays attached on IT8212 can be shown in system BIOS’s hard disk boot priority list. - Add detailed error information. - Add single mirrored drive check. When there is only one mirrored drive is attached on IT8212, show the following message “A disk member of this array was removed and it is not functional now. Please press <Y> to set up drive n as a IDE drive automatically or press OTHER KEY to setup array manually later.” - Change setup utility delete array's key [del] to key [d]. | 06/02/’04 |